.HE 6502 Cross Assembler Version 1.2

6502 Cross Assembler Version 1.2

User Manual Release 1.0

(C) B.O'Shaughnessy Software October 1984

All Rights Reserved

B.O'Shaughnessy

102 Whitethorn Avenue Yiewsley

West Drayton Middlesex UB7 8LB

Telephone (0895) 421197

MS-DOS is a trademark of Microsoft Corporation

CP/M is a trademark of Digital Research

PC-DOS is a trademark of I.B.M.

.FO (C) Copyright B.O'Shaughnessy October 1984       Page No #

.PA

Table of contents

Chapter   1.        The Assembler, and introduction.

Chapter   2.        The Command Line.

Chapter   3.        Command Line Switches.

Chapter   4.        Default command line switch settings.

Chapter   5.        Command Line Examples.

Chapter   6.        Numbers.

Chapter   7.        Operators.

Chapter   8.        Symbols

Chapter   9.        Directives Or Pseudo Operations.

Chapter   10.       Instructions.

Chapter   11.       Error Messages.

Chapter   12.       Comments.

Chapter   13.       6502 Test Programme listing.

Chapter   14.       End Of Assembly Messages.

.PA

Chapter 1        The Assembler and Introduction

This   manual  is  written  for  experienced  assembly   language

programmers and is not intended as a tutorial.  However there are

examples given throughout the manual to assist the programmer.

This manual assumes a knowledge of the computer and its operating

system  (MS-DOS,  PC-DOS  or  CP/M-86).  Source files  should  be

prepared with a text editor,  either Edlin or an alternative such

as Pmate.

This assembler is written in 100% 8086/8088 assembly language for

maximum speed and reliability.  It can assemble approximately 64K

of source code per minute,  this can be achieved using a Sirius 1

with  5.25 inch floppy disks.  On a test with 0.25 Mego  byte  of

source  code assembled to 19K of object code an assembly time  of

3.5 minutes was achieved.

This  assembler produces object code which can either be directed

to  the  current  list  device  for  down-loading  into   another

computer, or stored in an object file on disk.

A  source code listing and alphabetically sorted symbol table are

also  produced by the assembler,  which can be stored in a  print

file or directed to the console and printer.

Every  line of source code must end with a return followed  by  a

line feed. The source file must finish with an end of file marker

"^Z".,  which  will  be  handled  by  the  text  editor  or  word

processor.

.PA

Chapter 2               The Command Line

Below is the command line format for this assembler. The angled

brackets indicate optional parameters.

<drive:>6502COMP <drive:>SOURCE<.extension> <output:> <switches>

<drive:> This letter indicates the drive on which 6502COMP.COM is

to be found. If no drive is specified then the default drive will

be assumed.

<.extension>  Any  three  letter  file  name  extension.   If  no

extension  is specified then the assembler will assume  .SRC.  If

just the full stop is entered then no extension will be assumed.

<output:>  This  can be any letter indicating the drive to  which

output  files are directed.  If no output is specified  then  the

default drive will be assumed.

<switches>  These  can be any combination of the  nine  available

switches  defining assembly time options.  The Hard copy (H)  and

Download  (D) switches should not be used together as  both  send

data  via the primary list device.  If no switches are  specified

then automaticly the default switch settings will be used.

.PA

Chapter 3           The Command Line Switches

If  the (B)yte switch is enabled the assembler will include   all

code from the source file in the source code listing. Otherwise a

maximum  of  three  bytes from every line will  be  assumed.  For

example  the line (  BYT  "ABCDEFG") will only display the  first

three bytes "ABC" as "414243" in the source code listing.

If the (C)onsole switch is enabled then any text output from  the

assembler  will  be sent to the console.  This is useful  if  the

source  code listing or symbol table is required to be sent to  a

print file and displayed on the console simultaneously.

If  the (D)ownload switch is enabled all object code will be sent

to  the  current  list device.  This saves time  and  disk  space

because  it eliminates the need to create an object  file  before

downloading the object code into another computer. However, it is

essential  to check your source code for errors before using this

facility as any incorrect object code will  also be downloaded.

If  the (H)ard copy switch is enabled then all text  output  from

the assembler will be sent to the current list device.

If  the (L)ist switch is enabled then a source code listing  will

be generated by the assembler.

If  the (O)bject switch is enabled an object file will be created

by the assembler.

If  the  (P)rint switch is enabled a print file on disk  will  be

created by the assembler.

If the (S)ymbol table switch is enabled an alphabetically  sorted

symbol table will be generated by the assembler.

If  the  (W)ait  switch is enabled the assembler will  print  the

message  "Press  any key when ready" each time before  opening  a

source file. This is useful when the LNK pseudo operation is used

to link source files on different disks.  If this switch is  used

both  the object and print files must be directed to a  different

drive.  Thus if source files are held on drive B: and the disk in

drive  B:  is changed the object and print files must be held  on

drive A:

.PA

Chapter 4     Default Command Line Switch Settings.

For switches not explicitly set by the user in the command  line,

the assembler will assume the following cases :-

The (B)yte switch is enabled,  no more than three bytes of object

code  from every line of source code will be present in the  list

file.

The (C)onsole switch is enabled, all text output by the assembler

will be sent to the console.

The  (D)ownload switch is disabled.  The object code will not  be

output to the current list device.

The  (H)ard  copy  switch  is  disabled,  text  output  from  the

assembler will not be sent to the current list device.

The  (L)ist  switch  is disabled,  a source code listing will not

be generated by the assembler.

The  (O)bject  file switch  is enabled,  an object file  will  be

generated by the assembler.

The  (P)rint  file  switch  is enabled,  a print file will not be

generated by the assembler.

The  (S)ymbol table switch is enabled. A symbol table will not be

generated by the assembler.

The  (W)ait  switch  is disabled.  The assembler  will  not  wait

before opening source files.

.PA

Chapter 5             Command Line Examples

6502COMP SOURCE

The  source code file is assembled from the default disk  and  is

expected to have an extension of .SRC.  All output is directed to

the default disk and the default switch settings are used.

A:6502COMP SOURCE.SRC B:

The source code file is assembled from the default drive with the

extension  of .SRC using the assembler from drive A:.  All output

is directed to drive B: and the default switch setting are used.

A:6502COMP C:SOURCE.X B:

The  source  code  file is assembled  from  drive   C:  with  the

extension of .X  using the assembler from drive A:. All output is

directed to drive B: and the default switch settings are used.

A:6502COMP C:SOURCE.X B: OL

The source code file is assembled in the same manner as the  last

example  but without an object file or source code listing  being

generated by the assembler.

A:6502COMP C:SOURCE.X B: PSD

In this instance  no print file or symbol table will be generated

by  the  assembler  and the object code will  be  output  to  the

current list device.

A:6502COMP C:SOURCE.X B: HWCB

In  this case the text output from the assembler will be directed

to the current list device but not to the console.  Every byte of

object code will be displayed in the source code listing and  the

assembler  will also print the message "Press any key when ready"

before opening source code files.

.PA

Chapter 6                    Numbers

Numbers can be expressed in several ways the default is decimal.

12345

Hexadecimal numbers must be prefixed with a ($) dollar sign.

$10FA

Octal numbers must be prefixed with  an (@) at sign.

@173

Binary numbers must be prefixed with a (%) percent sign.

%1001101000110

Ascii characters must be prefixed with an (') apostrophe.

'A

The program counter is expresed by the (*) asterisk.

*+12

The high byte of a number can be obtained by preceding the number

with a > greater than symbol.

>$1234    =    $12

The low byte of a number can be obtained by preceding the  number

with a < less than symbol.

<$1234    =    $34

All  numbers are in modulo 65536 and no overflow or underflow  is

checked by the assembler.

65535+1   =    0

0-1       =    65535

.PA

Chapter 7                    Symbols

Numbers  can also be represented by symbols.  Symbols can be  any

number  of  characters in length but only the first fourteen  are

recognised by the assembler. Only alphanumeric characters and the

underline  character  can  be used in symbols.  Upper  and  lower

case  characters are treated as identical by the  assembler.  All

symbols  must start with an alphabetic character or the underline

character  otherwise the error message "Symbols must  start  with

alphabetic characters" will be generated.

LABEL_01 is valid but

01_LABEL is not

.PA

Chapter 8                   Operators

The  four  basic operators (+-*/) can be used in expressions  and

all  four  operators have the same  precedence.  Expressions  are

evaluated  from left to right and any combination of numbers  and

operators can by used.

%1010101+$12-<@712/'!*symbol+65535

.PA

Chapter 9        Directives or pseudo operations

The ORG pseudo operation

The  ORG pseudo operation is used to set the location counter  to

the  value  in the operand field and  may be repeated  throughout

the source code.  No forward references may be used as this might

cause Phase errors during pass 2 of assembly.

ORG  $2000

The EQU pseudo operation

The  EQU  pseudo  operation is used to equate the symbol  in  the

symbol field to the expression in the operand field.

SYMBOL    EQU  $1234

The  EQU  pseudo  operation cannot be used  to  redefine  already

defined symbols.  If this is attempted an "Over defined  symbol."

error will be generated when the programme is assembled.

The RES pseudo operation

The  RES pseudo operation is used to reserve a number of bytes in

memory.  The first value in the operand field contains the number

of bytes to reserve.  The second value is optional and if present

defines the value of the bytes to be reserved otherwise a default

value  of zero is assumed.  No forward references may be used  as

this might cause Phase errors during pass 2 of assembly.

RES  1024,12

The  above example defines an area of 1024 bytes  containing  the

value 12

.PA

The BYT pseudo operation

The  BYT pseudo operation is used to reserve bytes of memory with

the values specified in the operand field. Single bytes separated

by  commas  or strings of characters enclosed in  quotes  may  be

defined.  To  use a quote inside a string of characters place two

quotes one beside the other.  All Values must be contained within

a byte (0-255) any value outside this range will cause a  "Number

too large." error when the programme is assembled.

BYT  $12*%101,"characters string",@1,'X,SYMBOL

The WOR pseudo operation

The WOR pseudo operation is used to reserve words of two bytes in

memory  (low byte first) with the values specified in the operand

field. All values must be separated by commas.

WOR  %11111111111,$1234,65535

The DBY pseudo operation

The DBY pseudo operation is used to reserve words of two bytes in

memory  with  the values specified in  the  operand  field.  This

operation  is similar to the WOR psudo operation except the words

are  stored  high byte first.  All values must  be  seperated  by

commas.

DBY  $1000,$2000,$3000

.PA

The END pseudo operation

The  END pseudo operation is used to mark the end of  the  source

code  and  is optional except when the LNK pseudo  operation  has

been  used to link source files together.  In this case the  last

file must contain an END pseudo operation followed by the name of

the first source file.

END  B:FILENAME.SRC

All  lines  of  code after the END pseudo  operation  are  totaly

ignored by the assembler. If the END pseudo operation is not used

then assembly stops when the end of file marker "^Z" is found.

The LNK pseudo operation

The LNK pseudo operation is used to link source files together at

assembly time.  Any number of source files can be linked together

but  the last file must have an END pseudo operation followed  by

the name of the first source file

LNK  A:NEXTFIEL

The LON and LOF pseudo operations

The  LON and LOF pseudo operations are used to enable and disable

the  source code listing during assembly.  Thay also override the

state of the L switch.

.PA

Chapter 10                 Instructions

Branch instructions

BCC       Branch if carry clear (C=0)

BCS       Branch if carry set (C=1)

BNE       Branch if not equal to zero (Z=0)

BEQ       Branch if equal to zero (Z=1)

BPL       Branch if plus (S=0)

BMI       Branch if minus (S=1)

BVC       Branch if overflow clear (V=0)

BVS       Branch if overflow set (V=1)

Load and Store instructions

LDA       Load accumulator from memory

LDX       Load index register X from memory

LDY       Load index register Y from memory

STA       Store accumulator in memory

STX       Store index register X in memory

STY       Store index register Y in memory

Push and Pop instructions

PHA       Push accumulator onto stack

PHP       Push status register (p) onto stack

PLA       Pull contents of accumulator from stack

PLP       Pull contents of status register (p) from stack

Compare instructions

CMP       Compare memory with accumulator

CPX       Compare index register X with memory

CPY       Compare index register Y with memory

.PA

Arithmetic, shift, rotate and bit instructions

ADC       Add memory with carry to accumulator

AND       And memory with accumulator

EOR       Exclusive-or accumulator with memory

ORA       Logically or memory with accumulator

SBC       Subtract memory from accumulator with borrow

BIT       Bit test

ASL       Shift accumulator or memory byte left

LSR       Logical shift right of accumulator or memory

ROL       Rotate accumulator or memory left through carry

ROR       Rotate accumulator or memory right through carry

Status register manipulating instructions

CLC       Clear carry

CLD       Clear decimal mode

CLI       Clear interupt mask (enable interupts)

CLV       Clear overflow

SEC       Set carry

SED       Set decimal mode

SEI       Set interupt mask (disable interupts)

Jump and return instructions

JMP       Jump via absolute or indirect addressing

JSR       Jump to subreotine

RTI       Return from interupt

RTS       Return from subroutine

Register move instructions

TAX       Move from accumulator to index register X

TAY       Move from accumulator to index register Y

TSX       Move from stack pointer to index register X

TXA       Move from index register X to accumulator

TXS       Move from index register X to stack pointer

TYA       Move from index register Y to accumulator

.PA

Increment and decrement instructions

INC       Increment memory (by 1)

INX       Increment index register (by 1)

INY       Increment index register (by 1)

DEC       Decrement memory (by 1)

DEX       Decrement index register X (by 1)

DEY       Decrement index register Y (by 1)

Break instruction

BRK       Force break (trap or software interupt)

No operation instruction

NOP       No operation

.PA

Chapter 11               Error Messages.

There  are three different types of error message issued  by  the

assembler: "fatal", "non fatal" and "warning". All error messages

will be sent to the console overriding the state of the C switch.

Fatal  errors will cause assembly to be aborted and control  will

return to the operating system.

Non fatal errors and warning errors do not halt assembly, but the

line in  which the error occurred will be printed on the  console

followed by the error message.

.PA

Fatal Errors

File Not Found.

The source file cannot be found by the assembler.

Cannot Read Source File.

An  error has occurred while trying to read a block of code  from

the  source  file.  This can occur when assembling a source  file

without an end of file marker "^Z"

Disk Or Directory Full.

Either the disk or the directory is full.

Disk Full.

The disk is full.

Symbol Table Overflow.

The available memory is insufficent to hold the symbol table.

File Name Missing.

The file name following the LNK or END pseudo operation has  been

omitted from the source code.

.PA

Non Fatal Errors

Number Too Large.

A value greater then 255 has been used in the operand field of  a

BYT pseudo operation.

Out Of Range.

An  address greater than 255 has been found in the operand  field

of an instruction using zero page addressing.

Syntax Error.

An error has been found when trying to evaluate an expression

Illegal Symbol.

A  character  has  been  used in the symbol  field  that  is  not

alphanumeric or an underline.

Illegal Addressing.

The  instruction  found in the opcode field cannot be  used  with

this addressing mode.

Branch Out Of Range.

This  error occures when a branch less than -126 or greater  than

129 bytes  has been found in the source code.

Symbols Must Start With Alphabetic Characters

All symbols must start with an alphabetic or underline character

.PA

Undefined Symbol.

A symbol has been included in the operand field of an instruction

that has not been defined elsewhere in the source code.

Illegal Operand.

The assembler cannot recognize the operand field.

Symbol Over Defined.

The symbol has already been defined elsewhere in the source code.

Symbols can only be defined once.

No Opcode.

The  opcode  field is missing.  All lines of source  code  except

blank  or  commented  lines  must contain  an  opcode  or  pseudo

operation.

Illegal Opcode.

The assembler cannot recognize the opcode.

Missing Symbol.

There must be a symbol with which to equate a value.

Divide By Zero.

A  division  by zero has occurred whilst trying  to  evaluate  an

expression.

Phase Error.

The value of the symbol has changed between pass one and pass two

of assembly.

.PA

Warning Errors

Warning bug In 6502 JMP ($xxFF).

There  is a bug in the 6502 MPU jmp indirect instruction when  an

address ending in FF hex is used.

Warning extra Character(s) On Line.

The  line of source code has been assembled correctly  but  extra

characters were found

Warning over Length Line >132 Characters.

A line greater than 132 characters in length has been found

.PA

Chapter 12                  Comments

Blank  lines  and   all characters remaining on a  line  after  a

semicolon  (that has not  been used as part of an expresion) will

be taken as comment and totaly ignored by the assembler.

.PA

Chapter 13          End Of Assembly Messages

End Of Assembly.

This  message  will  be printed directly after  the  source  code

listing and symbol table.

Errors Found  0

This  is the total number of non fatal and warning  errors  found

during assembly.

Location counter C014

The  final  value  of  the  location counter  is  also  given  in

hexadecimal.

Maximum Symbols 3528

The  assembler calculates the maximum number of symbols that  can

be used before assembly starts.  This calculation is based on the

ammount of free RAM memory in the Transient Program Area less the

length  of  the assembler,  each label required 16 bytes of  RAM.

e.g. On a system with 256K of RAM where 180K is available for the

user  and  the  assembler takes up 30K,  150K  is  available  for

symbols. 150K/16 = 9600 symbols.

Symbols Used  4

This is the total number of symbols used in the source code file.

When this number equals the maximum number of symbols allowed the

fatal  error  "Symbol  Table Overflow." will be  printed  on  the

console and assembly aborted.

.PA

Chapter 14             6502 Test Programme

6502COMP Cross Assembler MS-DOS Version 1.0

Copyright (C) B.O'Shaughnessy October 1984

All Rights Reserved. Serial No 1

0000		;****************************************

0000		;*					*

0000		;* 6502 Cross Assembler Test programme	*

0000		;* (C) B.O'Shaughnessy october 1984	*

0000		;* All Rights Reserved.			*

0000		;*					*

0000		;****************************************

0000		;

0000				ORG	256

0100		;

0100	00FF=	BYTE		EQU	255

0100	0080=	ZERO		EQU	128

0100		;

0100	69FF	ABSOLUTE	ADC	#BYTE

0102	6580			ADC	ZERO

0104	6D0001			ADC	ABSOLUTE

0107	7580			ADC	ZERO,X

0109	7D0001			ADC	ABSOLUTE,X

010C	798000			ADC	ZERO,Y

010F	790001			ADC	ABSOLUTE,Y

0112	6180			ADC	(ZERO,X)

0114	7180			ADC	(ZERO),Y

0116		;

0116	29FF			AND	#BYTE

0118	2580			AND	ZERO

011A	2D0001			AND	ABSOLUTE

011D	3580			AND	ZERO,X

011F	3D0001			AND	ABSOLUTE,X

0122	398000			AND	ZERO,Y

0125	390001			AND	ABSOLUTE,Y

0128	2180			AND	(ZERO,X)

012A	3180			AND	(ZERO),Y

012C		;

012C	0A			ASL	A

012D	0680			ASL	ZERO

012F	0E0001			ASL	ABSOLUTE

0132	1680			ASL	ZERO,X

0134	1E0001			ASL	ABSOLUTE,X

0137		;

0137	90C7			BCC	ABSOLUTE

0139		;

0139	B0C5			BCS	ABSOLUTE

013B		;

013B	F0C3			BEQ	ABSOLUTE

013D		;

013D	2480			BIT	ZERO

013F	2C0001			BIT	ABSOLUTE

0142		;

0142	30BC			BMI	ABSOLUTE

0144		;

0144	D0BA			BNE	ABSOLUTE

0146		;

0146	10B8			BPL	ABSOLUTE

0148		;

0148	00			BRK

0149		;

0149	50B5			BVC	ABSOLUTE

014B		;

014B	70B3			BVS	ABSOLUTE

014D		;

014D	18			CLC

014E		;

014E	D8			CLD

014F		;

014F	58			CLI

0150		;

0150	B8			CLV

0151		;

0151	C9FF			CMP	#BYTE

0153	C580			CMP	ZERO

0155	CD0001			CMP	ABSOLUTE

0158	D580			CMP	ZERO,X

015A	DD0001			CMP	ABSOLUTE,X

015D	D98000			CMP	ZERO,Y

0160	D90001			CMP	ABSOLUTE,Y

0163	C180			CMP	(ZERO,X)

0165	D180			CMP	(ZERO),Y

0167		;

0167	E0FF			CPX	#BYTE

0169	E480			CPX	ZERO

016B	EC0001			CPX	ABSOLUTE

016E		;

016E	C0FF			CPY	#BYTE

0170	C480			CPY	ZERO

0172	CC0001			CPY	ABSOLUTE

0175		;

0175	C680			DEC	ZERO

0177	CE0001			DEC	ABSOLUTE

017A	D680			DEC	ZERO,X

017C	DE0001			DEC	ABSOLUTE,X

017F		;

017F	CA			DEX

0180		;

0180	88			DEY

.PA

0181		;

0181	49FF			EOR	#BYTE

0183	4580			EOR	ZERO

0185	4D0001			EOR	ABSOLUTE

0188	5580			EOR	ZERO,X

018A	5D0001			EOR	ABSOLUTE,X

018D	598000			EOR	ZERO,Y

0190	590001			EOR	ABSOLUTE,Y

0193	4180			EOR	(ZERO,X)

0195	5180			EOR	(ZERO),Y

0197		;

0197	E680			INC	ZERO

0199	EE0001			INC	ABSOLUTE

019C	F680			INC	ZERO,X

019E	FE0001			INC	ABSOLUTE,X

01A1		;

01A1	E8			INX

01A2		;

01A2	C8			INY

01A3		;

01A3	4C8000			JMP	ZERO

01A6	4C0001			JMP	ABSOLUTE

01A9	6C0001			JMP	(ABSOLUTE)

01AC		;

01AC	208000			JSR	ZERO

01AF	200001			JSR	ABSOLUTE

01B2		;

01B2	A9FF			LDA	#BYTE

01B4	A580			LDA	ZERO

01B6	AD0001			LDA	ABSOLUTE

01B9	B580			LDA	ZERO,X

01BB	BD0001			LDA	ABSOLUTE,X

01BE	B98000			LDA	ZERO,Y

01C1	B90001			LDA	ABSOLUTE,Y

01C4	A180			LDA	(ZERO,X)

01C6	B180			LDA	(ZERO),Y

01C8		;

01C8	A2FF			LDX	#BYTE

01CA	A680			LDX	ZERO

01CC	AE0001			LDX	ABSOLUTE

01CF	B680			LDX	ZERO,Y

01D1	BE0001			LDX	ABSOLUTE,Y

01D4		;

01D4	A0FF			LDY	#BYTE

01D6	A480			LDY	ZERO

01D8	AC0001			LDY	ABSOLUTE

01DB	B480			LDY	ZERO,X

01DD	BC0001			LDY	ABSOLUTE,X

.PA

01E0		;

01E0	4A			LSR	A

01E1	4680			LSR	ZERO

01E3	4E0001			LSR	ABSOLUTE

01E6	5680			LSR	ZERO,X

01E8	5E0001			LSR	ABSOLUTE,X

01EB		;

01EB	EA			NOP

01EC		;

01EC	09FF			ORA	#BYTE

01EE	0580			ORA	ZERO

01F0	0D0001			ORA	ABSOLUTE

01F3	1580			ORA	ZERO,X

01F5	1D0001			ORA	ABSOLUTE,X

01F8	198000			ORA	ZERO,Y

01FB	190001			ORA	ABSOLUTE,Y

01FE	0180			ORA	(ZERO,X)

0200	1180			ORA	(ZERO),Y

0202		;

0202	48			PHA

0203		;

0203	08			PHP

0204		;

0204	68			PLA

0205		;

0205	28			PLP

0206		;

0206	2A			ROL	A

0207	2680			ROL	ZERO

0209	2E0001			ROL	ABSOLUTE

020C	3680			ROL	ZERO,X

020E	3E0001			ROL	ABSOLUTE,X

0211		;

0211	6A			ROR	A

0212	6680			ROR	ZERO

0214	6E0001			ROR	ABSOLUTE

0217	7680			ROR	ZERO,X

0219	7E0001			ROR	ABSOLUTE,X

021C		;

021C	40			RTI

021D		;

021D	60			RTS

021E		;

021E	E9FF			SBC	#BYTE

0220	E580			SBC	ZERO

0222	ED0001			SBC	ABSOLUTE

0225	F580			SBC	ZERO,X

0227	FD0001			SBC	ABSOLUTE,X

022A	F98000			SBC	ZERO,Y

022D	F90001			SBC	ABSOLUTE,Y

0230	E180			SBC	(ZERO,X)

0232	F180			SBC	(ZERO),Y

0234		;

0234	38			SEC

0235		;

0235	F8			SED

0236		;

0236	78			SEI

0237		;

0237	8580			STA	ZERO

0239	8D0001			STA	ABSOLUTE

023C	9580			STA	ZERO,X

023E	9D0001			STA	ABSOLUTE,X

0241	998000			STA	ZERO,Y

0244	990001			STA	ABSOLUTE,Y

0247	8180			STA	(ZERO,X)

0249	9180			STA	(ZERO),Y

024B		;

024B	8680			STX	ZERO

024D	8E0001			STX	ABSOLUTE

0250	9680			STX	ZERO,Y

0252		;

0252	8480			STY	ZERO

0254	8C0001			STY	ABSOLUTE

0257	9480			STY	ZERO,X

0259		;

0259	AA			TAX

025A		;

025A	A8			TAY

025B		;

025B	BA			TSX

025C		;

025C	8A			TXA

025D		;

025D	9A			TXS

025E		;

025E	98			TYA

025F		;

025F				END

ABSOLUTE       0100 BYTE           00FF ZERO           0080

End Of Assembly.

Errors Found  0

Location counter  025F

Maximum Symbols  3463

Symbols Used  3

.OP

Users copy

Licence Agreement

On  condition that you sign and return this Licence Agreement  to

B.O'Shaughnessy Software.  B.O'Shaughnessy Software hereby grants

you a non exclusive and non transferable licence to use the  copy

of the software 6502 Cross Assembler.  Failure to sign or failure

to return the agreement and still use the software is illegal.

I   agree  that  B.O'Shaughnessy  Software  shall  not  be   held

responsible  for  any consequential damages that I  may  possibly

incur  through the use of 6502 Cross Assembler,  whether  through

the fault of B.O'Shaughnessy Software or not.

Any breach of one or more of the provisions of this Agreement  by

me  shall constitute an immediate termination of this  Agreement.

Nevertheless,  I agree that in the event of such termination, all

provisions   of  this  Agreement  which  protect  the  rights  of

B.O'Shaughnessy Software shall remain in force.

I hereby acknowledge that I have read this Agreement,  understand

it and agree to be bound by its terms and conditions.

Name

Address

Serial number

Signature

.FO

.PA

Licence Agreement

On  condition that you sign and return this Licence Agreement  to

B.O'Shaughnessy Software.  B.O'Shaughnessy Software hereby grants

you  a non exclusive and non transferable licence to use the copy

of the software 6502 Cross Assembler.  Failure to sign or failure

to return the agreement and still use the software is illegal.

I   agree  that  B.O'Shaughnessy  Software  shall  not  be   held

responsible  for  any consequential damages that I  may  possibly

incur  through the use of 6502 Cross Assembler,  whether  through

the fault of B.O'Shaughnessy Software or not.

Any breach of one or more of the provisions of this Agreement  by

me  shall constitute an immediate termination of this  Agreement.

Nevertheless,  I agree that in the event of such termination, all

provisions   of  this  Agreement  which  protect  the  rights  of

B.O'Shaughnessy Software shall remain in force.

I hereby acknowledge that I have read this Agreement,  understand

it and agree to be bound by its terms and conditions.

Name

Address

Serial number

Signature

Please return this copy of the Licencing Agreement to

B.O'Shaughnessy

102 Whitethorn Avenue

Yiewsley West Drayton

Middlesex UB7 8LB

.PA