Z80 Cross Assembler Version 2.0

User Manual Release 1.1

Updated August 1986

(C) B.O'Shaughnessy Software November 1984

All Rights Reserved

B.O'Shaughnessy

1 St. Georges Drive

Toftwood Dereham

Norfolk NR19 1LQ

MS-DOS is a trademark of Microsoft Corporation

CP/M is a trademark of Digital Research

PC-DOS is a trademark of I.B.M.

Table of contents

Chapter   1.        The Assembler, and introduction.

Chapter   2.        The Command Line.

Chapter   3.        Command Line Switches.

Chapter   4.        Default command line switch settings.

Chapter   5.        Command Line Examples.

Chapter   6.        Numbers.

Chapter   7.        Operators.

Chapter   8.        Symbols

Chapter   9.        Directives Or Pseudo Operations.

Chapter   10.       Instructions.

Chapter   11.       Error Messages.

Chapter   12.       Comments.

Chapter   13.       End Of Assembly Messages.

Chapter 1        The Assembler and Introduction

This   manual  is  written  for  experienced  assembly   language

programmers and is not intended as a tutorial.  However there are

examples given throughout the manual to assist the programmer.

This manual assumes a knowledge of the computer and its operating

system  (MS-DOS,  PC-DOS  or  CP/M-86).  Source files  should  be

prepared with a text editor, either Edlin, an alternative such as

The Norton Editor or WordStar in Non Document mode.

This assembler is written in 100% 8086/8088 assembly language for

maximum speed and reliability.  It can assemble approximately 64K

of source code per minute,  this can be achieved using a Sirius 1

with 5.25 inch floppy disks.  On a test with 250K bytes of source

code  assembled to  19K of machine code,  an assembly time of 3.5

minutes was achieved.

This assembler produces machine code which can either be directed

to  the  current  list  device  for  down-loading  into   another

computer, or stored in an binary file on disk.

A  source code  listing and an alphabetically sorted symbol table

can also be produced  by the assembler,  which can be stored in a

print file, directed to the console or sent to a printer.

Every  line of source code must end with a CRLF and the file must

finish with an end of file marker, "^Z", which should be  handled

by the  text editor or word processor.  The assembler will assume

an  extension  of .Z80  if one isn't  supplied and  generates the

machine  code file with  the same name  as the source, but with a

.CPM extension.

Chapter 2               The Command Line

Below is the command line format for this assembler. The angled

brackets indicate optional parameters.

<drive:>Z80COMP <drive:>SOURCE<.extension> <output:> <switches>

<drive:> This letter indicates the drive on which Z80COMP.COM  is

to be found. If no drive is specified then the default drive will

be assumed.

<.extension>  Any  three  letter  file  name  extension.   If  no

extension  is specified then the assembler will assume  .Z80.  If

just the full stop is entered then no extension will be used.

<output:>  This  can be any letter indicating the drive to  which

output  files are directed.  If no output is specified  then  the

default drive will be assumed.

<switches>  These  can be any combination of the  nine  available

assembly  option switches, (POLCBSHWD).  The switches are toggles

and therefore  if the same  option is used  twice it  will cancel

itself out.  The assembler  starts with  the default settings and

applies the command line  switches,  therefore if no machine code

is required,  the (O) switch will turn the default off.  The Hard

copy (H) and Download (D) switches should not be used together as

both  send data via the primary list  device.  If no switches are

specified  then the  assembler will stay with the  default switch

settings, (BCO).

Chapter 3           The Command Line Switches

If  the (B)yte switch is enabled the assembler will include   all

bytes of machine code in the listing. Otherwise a maximum of four

bytes from every line  will be assumed. This should be sufficient

for machine code,  but will restrict  the display  of data bytes.

For example  the  line ( JP 1234H )  would  appear  completely as

"C33412"  whereas the  line ( DB  "ABCDEFG")  will be  limited to

"41424344" in the source code listing.

If the (C)onsole switch is enabled then any text output from  the

assembler  will  be sent to the console.  This is useful  if  the

source  code listing or symbol table is required to be sent to  a

print file and displayed on the console simultaneously.

If the (D)ownload switch is enabled all machine code will be sent

to  the  current  list device.  This saves time  and  disk  space

because  it  eliminates the need to create an binary file  before

downloading the binary code into another computer. However, it is

essential  to check your source code for errors before using this

facility as any incorrect machine code will  also be downloaded.

If  the (H)ard copy switch is enabled then all text  output  from

the assembler will be sent to the current list device.

If  the (L)ist switch is enabled then a source code listing  will

be generated by the assembler.

If  the (O)bject switch is enabled a binary file  will be created

by the assembler.

If  the  (P)rint switch is enabled a print file on disk  will  be

created by the assembler.

If the (S)ymbol table switch is enabled an alphabetically  sorted

symbol table will be generated by the assembler.

If  the  (W)ait  switch is enabled the assembler will  print  the

message  "Press  any key when ready" each time before  opening  a

source file. This is useful when the LNK pseudo operation is used

to link source files on different disks.  If this switch is used

both  the binary and print files must be directed to a  different

drive.  Thus if source files are held on drive B: and the disk in

drive  B:  is changed the binary and print files must be held  on

drive A:

Chapter 4     Default Command Line Switch Settings.

For switches not explicitly set by the user in the command  line,

the assembler will assume the following cases :-

The  (B)yte switch is enabled,  all the bytes of binary code from

every line of source code  will be present in the  list file.

The (C)onsole switch is enabled, all text output by the assembler

will be sent to the console.

The  (D)ownload switch is disabled. The machine code will not  be

sent to the current list device.

The  (H)ard  copy  switch  is  disabled,  text  output  from  the

assembler will not be sent to the current list device.

The  (L)ist switch is disabled, a source code listing will not be

output by the assembler.

The  (O)bject  file switch  is enabled,  a binary  file  will  be

generated by the assembler.

The  (P)rint  file switch is disabled,  a print file  will not be

generated by the assembler.

The  (S)ymbol table switch  is disabled.  A symbol table will not

be output by the assembler.

The  (W)ait  switch  is disabled.  The assembler  will  not  wait

before opening source files.

The default values can be  patched with a binary editor, the nine

command  switch  letters  are  stored  at  0111H followed  by the

default  values  at 011BH.  A value of 1 indicates  the switch is

enabled, whereis a value  of 0 indicates  it is disabled.

Chapter 5             Command Line Examples

Z80 SOURCE

The  source code file is assembled from the default disk  and  is

expected to have an extension of .Z80.  All output is directed to

the default disk and the default switch settings are used.

A:Z80COMP SOURCE.SRC B:

The source code file is assembled from the default drive with the

extension  of .SRC using the assembler from drive A:.  All output

is directed to drive B: and the default switch setting are used.

A:Z80COMP C:SOURCE.X B:

The  source  code  file  is assembled  from  DRIVE  C:  with  the

extension of .X  using the assembler from drive A:. All output is

directed to drive B: and the default switch settings are used.

A:Z80COMP C:SOURCE.X B: OL

The source code file is assembled in the same manner as the  last

example  but without a binary file and with a source code listing

being generated by the assembler.

A:Z80COMP C:SOURCE.X B: PSD

In this instance a  print file and symbol table will be generated

by  the  assembler  and the machine code will  be output  to  the

current list device.

A:Z80COMP C:SOURCE.X B: HWCB

In  this case the text output from the assembler will be directed

to the  current list device  but not to the console.  The machine

code will be limited to four bytes in the source code listing and

the  assembler  will also  print  the message "Press any key when

ready" before opening source code files.

Chapter 6                    Numbers

Numbers can be expressed in several ways, the default is decimal.

12345

Hexadecimal  numbers  must be suffixed by the letter  H,  if  the

first digit is not in the range 0..9 then a prefix of 0 must also

be added.  9A4H is a valid hexadecimal number but A94h is not, it

should be entered as 0A94H

Octal numbers must end with the letter (O) or (Q).

173O

173Q

Binary numbers must end with the letter (B).

1001101000110B

Ascii characters must be preceded by an (') apostrophe.

'A

The program counter is expressed by the ($) dollar.

$+12

The high byte of a number can be obtained by preceding the number

with a ">" greater than symbol.

>1234H   =    12H

The low byte of a number can be obtained by preceding the  number

with a "<" less than symbol.

<1234H    =    34H

All  numbers are in modulo 65536 and no overflow or underflow  is

checked by the assembler.

65535+1   =    0

0-1       =    65535

Chapter 7                    Symbols

Numbers  can also be represented by symbols.  Symbols can be  any

number  of  characters in length but only the first fourteen  are

recognised by the assembler. Only alphanumeric characters and the

underline  character  can  be used in symbols.  Upper  and  lower

case  characters are treated as identical by the  assembler.  All

symbols  must start with an alphabetic character or the underline

character  otherwise the error message "Symbols must  start  with

alphabetic characters" will be generated.

LABEL_01 is valid but

01_LABEL is not

Chapter 8                   Operators

The  four  basic operators (+-*/) can be used in expressions  and

all  four  operators have the same  precedence.  Expressions  are

evaluated  from left to right and any combination of numbers  and

operators can by used.

1010101B+12H-<712O/'!*symbol+65535

Chapter 9        Directives or pseudo operations

The ORG pseudo operation

The  ORG pseudo operation is used to set the location counter  to

the  value  in the operand field and  may be repeated  throughout

the source code.  No forward references may be used as this might

cause Phase errors during pass 2 of assembly.

ORG  2000H

The EQU pseudo operation

The  EQU  pseudo  operation is used to equate the  symbol  in  the

symbol field to the expression in the operand field.

SYMBOL    EQU  1234H

The  EQU  pseudo  operation cannot be used  to  redefine  already

defined symbols.  If this is attempted an "Over defined  symbol."

error will be generated when the programme is assembled.

The DS pseudo operation

The  DS pseudo operation is used to reserve a number of bytes  in

memory.  The first value in the operand field contains the number

of bytes to reserve.  The second value is optional and if present

defines the value of the bytes to be reserved otherwise a default

value  of zero is assumed.  No forward references may be used  as

this might cause Phase errors during pass 2 of assembly.

DS  1024,12

The  above example defines an area of 1024 bytes  containing  the

value 12

The DB pseudo operation

The  DB pseudo operation is used to reserve bytes of memory  with

the values specified in the operand field. Single bytes separated

by  commas  or  strings of characters enclosed in quotes  may  be

defined.  To use a quote inside a string of characters place  two

quotes one beside the other.  All Values must be contained within

a  byte (0-255) any value outside this range will cause a "Number

too large." error when the programme is assembled.

DB  12H*101B,"characters string",1Q,'X,SYMBOL

The DW pseudo operation

The  DW pseudo operation is used to reserve words of two bytes in

memory (low byte first) with the values specified in the  operand

field. All values must be separated by commas.

DW  11111111111B,1234H,65535

The END pseudo operation

The  END pseudo operation is used to mark the end of  the  source

code  and  is optional except when the LNK pseudo  operation  has

been  used to link source files together.  In this case the  last

file must contain an END pseudo operation followed by the name of

the first source file.

END  B:FILENAME.Z80

All  lines  of code after the END pseudo  operation  are  totally

ignored by the assembler. If the END pseudo operation is not used

then assembly stops when the end of file marker "^Z" is found.

The LNK pseudo operation

The LNK pseudo operation is used to link source files together at

assembly time.  Any number of source files can be linked together

but  the last file must have an END pseudo operation followed  by

the name of the first source file

LNK  A:NEXTFIEL

The LON and LOF pseudo operations

The  LON and LOF pseudo operations are used to enable and disable

the  source code listing during assembly.  They also override the

state of the L switch.

The INS pseudo operation

The  INS pseudo operation is used to insert library source  files

at  assembly time.  Any number of files can be inserted  but.  If

nesting  is attempted the fatal error "Cannot Nest Insert  Files"

will be displayed and assembly terminated.

Chapter 10                 Instructions

r         Any 8 Bit Register From B C D E H L A

dd        Any 16 Bit Register From BC DE HL SP

qq        Any 16 Bit Register From BC DE HL AF

s         Can Be Any From r n (HL) (IX+d) (IY+d)

m         Can Be Any From r (HL) (IX+d) (IY+d)

ss        Any 16 Bit Register From BC DE HL SP

pp        Any 16 Bit Register From BC DE IX SP

rr        Any 16 Bit Register From BC DE IY SP

b         Any Number From 0-7

cc        Any From NZ Z NC C PO PE P M

p         Any  Number From 00H 08H 10H 18H 20H 28H 30H 38H

8-Bit Load Group

LD        r,r'

LD        r,n

LD        r,(HL)

LD        r,(IX+d)

LD        r,(IY+d)

LD        (HL),r

LD        (IX+d),r

LD        (IY+d),r

LD        (HL),n

LD        (IX+d),n

LD        (IY+d),n

LD        A,(BC)

LD        A,(DE)

LD        A,(nn)

LD        (BC),A

LD        (DE),A

LD        (nn),A

LD        A,I

LD        A,R

LD        I,A

LD        R,A

16-Bit Load Group

LD        dd,nn

LD        IX,nn

LD        IY,nn

LD        HL,(nn)

LD        dd,(nn)

LD        IX,(nn)

LD        IY,(nn)

LD        (nn),HL

LD        (nn),dd

LD        (nn),IX

LD        (nn),IY

LD        SP,HL

LD        SP,IX

LD        SP,IY

PUSH      qq

PUSH      IX

PUSH      IY

POP       qq

POP       IX

POP       IY

Exchange Group And Block Transfer And Search Group

EX        DE,HL

EX        AF,AF'

EXX

EX        (SP),HL

EX        (SP),IX

EX        (SP),IY

LDI

LDIR

LDD

LDDR

CPI

CPIR

CPD

CPDR

8-Bit Arithmetic And Logical Group

ADD       A,r

ADD       A,n

ADD       A,(HL)

ADD       A,(IX+d)

ADD       A,(IY+d)

ADC       A,s

SUB       s

SBC       A,s

AND       s

OR        s

XOR       s

CP        s

INC       r

INC       (HL)

INC       (IX+d)

INC       (IY+d)

DEC       m

General Purpose Arithmetic And CPU Control Groups

DAA

CPL

NEG

CCF

SCF

NOP

HALT

DI

EI

IM        0

IM        1

IM        2

16-Bit Arithmetic Group

ADD       HL,ss

ADC       HL,ss

SBC       HL,ss

ADD       IX,pp

ADD       IY,rr

INC       ss

INC       IX

INC       IY

DEC       ss

DEC       IX

DEC       IY

Rotate And Shift Group

RLCA

RLA

RRCA

RRA

RLC       m

RL        m

RRC       m

RR        m

SLA       m

SRA       m

SRL       m

RLD

RRD

Bit Set, Reset And Test Group

BIT       b,r

BIT       b,(HL)

BIT       b,(IX+d)

BIT       b,(IY+d)

SET       b,r

SET       b,(HL)

SET       b,(IX+d)

SET       b,(IY+d)

RES       b,r

RES       b,(HL)

RES       b,(IX+d)

RES       b,(IY+d)

Jump Group

JP        nn

JP        cc,nn

JP        (HL)

JP        (IX)

JP        (IY)

JR        e

JR        C,e

JR        NC,e

JR        Z,e

JR        NZ,e

DJNZ

Call And Return Groups

CALL      nn

CALL      cc,nn

RET

RET       cc

RETI

RETN

RST       p

Input And Output Groups

IN        A,(n)

IN        r,(C)

INI

INIR

IND

INDR

OUT       (n),a

OUT       (C),r

OUTI

OTIR

OUTD

OTDR

Chapter 11               Error Messages.

There  are three different types of error message issued  by  the

assembler: "fatal", "non fatal" and "warning". All error messages

will be sent to the console overriding the state of the C switch.

Fatal  errors will cause assembly to be aborted and control  will

return to the operating system.

Non fatal errors and warning errors do not halt assembly, but the

line in  which the error occurred will be printed on the  console

followed by the error message.

Fatal Errors

File Not Found.

The source file cannot be found by the assembler.

Cannot Read Source File.

An  error has occurred while trying to read a block of code  from

the  source  file.  This can occur when assembling a source  file

without an end of file marker "^Z"

Disk Or Directory Full.

Either the disk or the directory is full.

Disk Full.

The disk is full.

Symbol Table Overflow.

The available memory is insufficent to hold the symbol table.

File Name Missing.

The file name following the LNK or END pseudo operation has  been

omitted from the source code.

Non Fatal Errors

Number Too Large.

A  value  has been found in the operand field too large  for  the

instruction or pseudo operation.

Syntax Error.

An error has been found when trying to evaluate an expression.

Illegal Symbol.

A  character  has  been  used in the symbol  field  that  is  not

alphanumeric or an underline.

Symbols Must Start With Alphabetic Characters

All symbols must start with an alphabetic or underline character

Undefined Symbol.

A symbol has been included in the operand field of an instruction

that has not been defined elsewhere in the source code.

Relative Jump Out Of Range

This error occurs when a jump less than -126 or greater than  129

bytes  has been found in the source code.

Illegal Relative Jump.

The  relative  jump  instruction cannot be used with  either  the

parity or the sign flag.

Illegal Operand.

The assembler cannot recognize the operand field.

Symbol Over Defined.

The symbol has already been defined elsewhere in the source code.

Symbols can only be defined once.

No Opcode.

The  opcode  field is missing.  All lines of source  code  except

blank  or  commented  lines  must contain  an  opcode  or  pseudo

operation.

Illegal Opcode.

The assembler cannot recognize the opcode.

Missing Symbol.

There must be a symbol with which to equate a value.

Divide By Zero.

A  division  by zero has occurred whilst trying  to  evaluate  an

expression.

Phase Error.

The value of the symbol has changed between pass one and pass two

of assembly.

Offset Out Of Range.

Offsets  used  with  the index registers "IX,IY" must be  in  the

range of -128 to +127

Warning Errors

Warning extra Character(s) On Line.

The  line of source code has been assembled correctly  but  extra

characters were found

Warning over Length Line >132 Characters.

A line greater than 132 characters in length has been found

Chapter 12                  Comments

Blank  lines  and   all characters remaining on a  line  after  a

semicolon  (that has not  been used as part of an expresion) will

be taken as comment and totally ignored by the assembler.

In  using  the  IY and IX registers with an offset  of  zero  the

offset must still be explicitly declared i.e. (IY+0) and (IX+0).

Chapter 13          End Of Assembly Messages

End Of Assembly.

This  message  will  be printed directly after  any  source  code

listing and symbol table, if requested, and is accompanied by the

following informational messages.

Errors Found  0

This  is the total number of non fatal and warning  errors  found

during assembly.

Location counter C014

The final value of the location counter  is given in hexadecimal.

Maximum Symbols 3528

The  assembler calculates the maximum number of symbols that  can

be used before assembly starts.  This calculation is based on the

ammount of free RAM memory in the Transient Program Area less the

length  of  the assembler,  each label required 16 bytes of  RAM.

e.g. On a system with 256K of RAM where 180K is available for the

user  and  the  assembler takes up 30K,  150K  is  available  for

symbols. 150K/16 = 9600 symbols.

Symbols Used  4

This is the total number of symbols used in the source code file.

When this number equals the maximum number of symbols allowed the

fatal  error  "Symbol  Table Overflow." will be  printed  on  the

console and assembly aborted.